StaNDARD OPERATING PROCEDURE

Alizarin Red staining of monolayers

| Created by: |  | reference number: |  |
| --- | --- | --- | --- |
| David Cook |  | SOP 25 |  |
| date: |  |  |
| 16/11/2010 |  |  |  |

- Wash cells once with PBS

- Fix cells for 20 minutes in 4% PFA

- Wash cells tree times in PBS

- Stain cells with 40mM Alzarin Red S solution (pH4.2) for 20min at RT

- Wash cells three times in PBS, followed by washes in tap water, to remove non-specific staining.

- Allow plates to dry prior to imaging.